EMFISIS  COMMAND and TELEMETRY PACKET FORMATS

Table of Contents

1.	Introduction	4

2.	Housekeeping	5

2.1	Overview	5

2.2	Format	6

3.	Magnetometer	7

3.1	Overview	7

3.2	Uncompressed Magnetometer data.	8

3.3	Compressed Magnetometer data.	9

3.3.1	Subpacket Format	10

3.4	Magnetometer Data Decompression	11

3.4.1	Magnetometer Data Decompression Algorithm	12

3.5	Magnetometer Data Compression Algorithm	15

3.6	FAILSAFE Magnetometer data.	16

4.	Waves Packets	17

4.1	Overview	17

4.2	General Waves Format	17

5.	Miscellaneous Packets	19

5.1	Memory Dump	19

5.2	Waves Dump	20

5.3	LVPS packet	21

5.4	Space Weather packet	22

5.5	OSP	22

6.	Commands	23

6.1	Overview	23

6.2	Time and Status	23

6.3	EMFISIS Telecommands	25

6.4	NOP	26

6.5	Reset	27

6.6	Write Enable	28

6.7	Peek	29

6.8	Poke	30

6.9	Jump	30

6.10	MagPacketType	31

6.11	UploadAbort	32

6.12	UploadData	33

6.13	Upload Start	34

6.14	MemoryDump	35

6.15	WavesDump	36

6.16	EnableWavesInterrupt	37

List of Tables and Figures

Table 1 RBSP  ITF Format	4

Table 2 EMFISIS Generic Packet Format	5

Table 3 EMFISIS Housekeeping Packet Format	6

Table 4 Uncompressed Magnetometer Data Packet Format	8

Table 5 Compressed Magnetometer Data Packet Format	9

Table 6 Subpacket format :  (example with two compressed subpackets – first with 10 samples , second with 12)	10

Table 7  Compression/Decompression (N=0  to # Samples -1)	11

Table 8 FAILSAFE Magnetometer Data Packet Format	16

Table 9 EMFISIS Waves Packet Format	17

Table 10 WAVES Packet Types	18

Table 11 WAVES Header	18

Table 12 Memory Dump Packet Format	19

Table 13 WAVES Dump Packet	20

Table 14 LVPS Packet	21

Table 15 Space Weather Packet	22

Table 16 Time and Status Message	23

Table 17 Generic EMFISIS Telecommand	25

Table 18 NOP	26

Table 19 Reset	27

Table 20 Write Enable	28

Table 21 Peek	29

Table 22 Poke	30

Table 23 MagPacketType	31

Table 24 UploadAbort	32

Table 25 UploadData	33

Table 26 UploadStart	34

Table 27 MemoryDump	35

Table 28 WavesDump	36

Table 29 EnableWavesInterrupt	37

Figure 1 Magnetometer Data Decompression Flowchart	14

REVISION NOTICE

| Version Identifier | Date of Issue | Summary of Changes |
| --- | --- | --- |
| 1.0 | 1/2/2009 | Initial releas3 |
| 1.1 | 1/6/2009 | Corrected decomp. Description – added flowchart – define range bits |
| 1.2 | 1/6/2009 | Correct Flow chart |
| 2.0 | 2/3/2009 | Expand to include all packet types |
| 2.1 | 3/10/2009 | Revise Wave packet Format |
| 2.2 | 3/17/2009 | revise HK Packet |
| 2.3 | 3/25/2009 | Revise Waves packets – add Shared data packet, LVPS packet, add commands |
| 2.4 | 3/25/2009 | revise RESET/Write Enable Commands, modify LVPS packet |
| 2.5 | 3/29/2009 | Revise HK – combine Shared Data into HK |
| 2.6 | 4/3/2009 | Revise HK – Add Space Weather |
| 2.7 | 4/24/09 | Revise MAg coord system references and Waves Packet definition |

# Introduction

Table 1 RBSP  ITF Format

| Field Name | Length | Description |
| --- | --- | --- |
| Sync Pattern | 32 bits | Thirty-two (32) bit sync word containing a fixed data value of 0xfefa30c8 |
| Aliveness Status | 1 bit | Aliveness Status (heartbeat) for Autonomy (toggle each second), (0 for commands) |
| Power Down Request | 1 bit | Instrument power off request for Autonomy (Assert to be powered OFF), (0 for commands) |
| Spare | 1 bit | Spare Bit (Always 0) |
| Message Length | 13 bits | Number of bytes following the “Message Length” field, including the “Checksum” field. The value of this field should always be EVEN. Range for the value of this field is (4 – 8190) for telemetry and (4-506) for commands. |
| First Packet Header Index | 16 bits | Offset of first packet header in the ITF (0 – 4094) location of first byte of packet within ITF 0xFFFF – No packet header in this ITF Commands always 0 for the command ITF |
| CCSDS Packetized Message Data | 0-8186 Bytes | S/C time and status, commands, shared data, magnetic field data, housekeeping or science data packets. |
| Checksum | 16 bits | Sixteen (16) bit exclusive-OR of all data following the Sync Pattern (seeded by 0x0000) |

Table 2 EMFISIS Generic Packet Format

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 280- 2BF (hex) | EMFISIS Instrument Allocated message APID |
| Grouping Flags | 2 | 3 | 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | 3 – 4089 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| Data | 2 – 4084 Bytes | Variable | EMFISIS Specific telemetry data |

# Housekeeping

# Overview

# Format

Table 3 EMFISIS Housekeeping Packet Format

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 280- 2BF (hex) | EMFISIS Instrument Allocated message APID |
| Grouping Flags | 2 | 3 | 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | 33 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| Critical HK |  |  |  |
| CDPU State | 16 |  | Status of CDPU error counters |
| Operating State | 4 | 0-15 | 15=AHBROM 14=POST 1-13 – TBD 0=nominal |
| spare | 4 |  | TDB |
| A/D Chan 0 | 8 |  | +5V Current |
| A/D Chan 1 | 8 |  | 3,3V Current |
| A/D Chan 2 | 8 |  | 1.8V Current |
| A/D Chan 3 | 8 |  | +13V Current |
| A/D Chan 4 | 8 |  | -13 V Currrent |
| Commands Accepted | 8 | 0-256 |  |
| Commands Rejected | 8 | 0-256 |  |
| Last Command | 16 |  | Opcode and Flags |
| CDPU Status | 28 |  |  |
| SubCom Counter | 4 | 0-15 | always 0 in AHBROM |
| CDPU Status | 32 |  | subcom 8 |
| A/D Chan 5-8 | 8 |  | subcom 4 |
| A/D Chan 9-16 | 8 |  | subcom 8 |
| A/D Chan 17-24 | 8 |  | Subcom 8 |
| Spare | 8 |  |  |
| Shared Data |  |  |  |
| Magnetometer zero crossing time (MET Seconds) | 8 | 0-0xff | Least Significant Byte of MET |
| Magnetometer zero crossing time (MET subseconds) | 8 | 0-0x9c | 6.4 milliseconds/bit |
| Magnetometer Range | 2 | 0-3 | 00: 256 nT 01: 4096 nT 10: not used 11: 65536 nT |
| EMFISIS flags | 14 |  | TBD |

# Magnetometer

# Overview

During Normal Operations EMFISIS will produce one Magnetometer packet every 4 seconds.  The packet will contain either uncompressed or compressed data. For testing purposes, both packet types may be generated simultaneously. Compressed or uncompressed data may be enabled via Telecommand. The default condition will be to generate uncompressed data.

Section 6 describes an alternative packet format to be generated if EMFISIS is operating in FAISLAFE Mode.

During Normal Operations each CCSDS Packet contains a  CCSDS Primary Header,  Secondary Header (MET Seconds) MET Subseconds, Data header and 768 16 bit values of magnetometer data, 256 U , 256 V and 256 W component measurements. (See Tables 1 and 2) The data is organized such that the 256 U values are first, followed by the 256 V and 256 W values.  If the values are extracted into 256 element arrays U[i], V[i],W[i] (i=0 to 255)  then the magnetometer vector corresponding to the packet time stamp will be the set of U[0],V[0],W[[0]. The time for each subsequent vector will be:

Sample Time = (Packet Time Stamp + i*(.015625))

(.015625 = 1/64 since there are 64 samples/second).

# Uncompressed Magnetometer data.

Table 4 Uncompressed Magnetometer Data Packet Format

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 290 (hex) | EMFISIS APID |
| Grouping Flags | 2 | 3 | CCSDS definition of grouping flags: 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | 1545 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| Cal Sate | 1 | 0= off 1= on |  |
| Range | 2 | 0-3 | 00: 256 nT 01: 4096 nT 10: not used 11: 65536 nT |
| Spare | 29 | 0 |  |
| Raw Mag U (256 samples) | 256*16 | -32768 - 32767 |  |
| Raw Mag V (256 samples) | 256*16 | -32768 - 32767 |  |
| Raw Mag W (256 samples) | 256*16 | -32768 - 32767 |  |

- Packets will be generated every 4 seconds.

- The time tag will correspond to the first data sample in the packet.

- Multi-byte data quantities are transmitted Most Significant Byte First in the CCSDS Packet.

# Compressed Magnetometer data.

When enabled, EMFISIS will produce apply a lossless compression algorithm to the Magnetometer data. The packet format is described in Tables 2 and 3.  The decompression algorithm is described in Sectio 4.

Table 5 Compressed Magnetometer Data Packet Format

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 292 (hex) | EMFISIS APID |
| Grouping Flags | 2 | 3 | CCSDS definition of grouping flags: 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | Variable | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| Cal Sate | 1 | 0= off 1= on |  |
| Range | 2 | 0-3 | 00: 256 nT 01: 4096 nT 10: not used 11: 65536 nT |
| Spare | 29 | 0 |  |
| Compressed Mag U Subpackets (256 samples) | Variable | N/A | see Table 3 |
| Compressed Mag V Subpackets (256 samples) | Variable | N/A | see Table 3 |
| Compressed Mag W Subpackets (256 samples) | Variable | N/A | See Table 3 |

- Packets will be generated every 4 seconds.

- The time tag will correspond to the first data sample in the packet.

- The U,V,W component data are compressed and will produce variable length packets.  The compression/decompression algorithms are is described in Section 4.

- Multi-byte data quantities are transmitted Most Significant Byte First in the CCSDS Packet.

## Subpacket Format

The magnetometer data is compressed into a set of subpackets.   The subpackets  are of variable length and must be decompressed in order to reproduce the original data stream.  The number of samples in a subpacket is determined by the compression algorithm and will vary. The maximum number of samples in any subpacket is 128.  Each subpacket will be padded to occupy an integral number of 32 bit words. The packet format is shown in Table 3. Reconstruction of the original data is shown in Table 4. After reconstruction, each CCSDS packet should yield 768 samples of uncompressed data corresponding to the original sets of 256 U, 256 V and 256 W components. The transition between components will always be at a subpacket boundary, that is, a subpacket will not contain more than one component type.

Table 6 Subpacket format :  (example with two compressed subpackets – first with 10 samples , second with 12)

| byte/word | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- |
| 0 | Nsamp=10 | 0 | X[0][8:15] | X[0][0:7] |
| 1 | C[3] | C[2] | C[1][8:15] | C[1][0:7] |
| 2 | C[7] | C[6] | C[5] | C[4] |
| 3 | pad | pad | C[9] | C[8] |
| 4 | Nsamp=12 | 0 | X[0][8:15] | X[0][0:7] |
| 5 | C[3] | C[2] | C[1][8:15] | C[1][0:7] |
| 6 | C[7] | C[6] | C[5] | C[4] |
| Nwords -1 | C[11] | C[10] | C[9] | C[8] |

Notes

  - bits 16-23 of first 32 bit word are always 0

  - pad bytes of 0 are appended to the end of the data to make the subpacket an integral number of 32 bit words.

  - Multi-byte data quantities are transmitted Most Significant Byte First in the CCSDS Packet.

    - The table is organized by 32 bit word ( byte numbering is WRT the 32 bit word – not packet byte order).

# Magnetometer Data Decompression

The correspondence between  the Original data, the Compressed data and the Decompressed data is shown in Table 4.

Table 7  Compression/Decompression (N=0  to # Samples -1)

| Original data value | Compressed Data | Intermediate value | Decompressed |
| --- | --- | --- | --- |
| X[0] | C[0] = X[0] |  | X[0] = C[0] |
| X[1] | C[1] = X[1] – X[0] | S[1] = X[1] - X[0] | X[1] = X[0] + C[1] |
| X[2] | C[2] = X[1] + S[1] - X[2] | S[2] = X[2] - X[1] | X[2] = X[1] + S[1] – C[2] |
| X[3] | C[3] = X[2] + S[2] – X[3] | S[3] = X[3] - X[2] | X[3] = X[2] + S[2} - C[3] |
| X[N] | C[N] = X[N-1] +S[N-1] – X[N] |  | X[N] = X[N-1] + S[ N-1] - C[N] |

Note:

- In Table X , the element of the array X are always 16 bit values. The array C represents the Compressed data and is stored in the output subpacket as either a 16 bit or 8 bit value as described below. (C[0] and C[1] are 16 bit quantities, C[2] … C[N] are 8 bit quantities.

## Magnetometer Data Decompression Algorithm

Given a CCSDS packet containing Compressed Magnetometer data, extract the packet data size primary header. The compressed data begins 10 bytes after the  primary header or at the 16th byte of the CCSDS Packet. We need to know the number of 32 bit words to process. All Compressed magnetometer data packets will contain an integral number of 32 bit data words.

- Calculate Nwords (number of 32 bit words) Nwords =  (Packet Data Length -9)/4

  - This subtracts the size of Time Tag and Data header (10 Bytes)

  - If Nwords = 0 (empty packet)

    - This is an error condition: Empty packets should not be generated.

The Compressed data begins at the 16th byte of the CCSDS Packet.

- read 32 bit word

  - Multi-byte data quantities are transmitted Most Significant Byte First in the CCSDS Packet.

- extract Nsamp from bits 24-31

- extract first data value X[0] from bits 0-15

- decrement Nsamp

- decrement Nwords

- If Nwords = 0 go to End of CCSDS Packet

    - If Nsamp = 0 Go to to Step 2

  - read next 32 bit word

  - extract Slope from bits 0-15

  - Uncompress X[1]= X[0] + Slope

  - decrement Nsamp

    - if Nsamp = 0

      - Decrement Nwords

      - If Nwords =0 Go To End Of CCSDS Packet

      - return to Step 2

  - extract compressed value C from bits 16-23; sign extend into 16 bit value

  - Uncompress X[2] = X[1] + Slope – C

  - Recalculate Slope = X[2] – X[1]

  - decrement Nsamp

    - if Nsamp = 0

      - Decrement Nwords

      - If Nwords =0 Go To End Of CCSDS Packet

      - return to Step 2

  - extract compressed value C from bits 24-31; sign extend into 16 bit value

  - Uncompress X[3] = X[2] + Slope – C

  - Recalculate Slope = X[3] – X[2]

  - decrement Nsamp

    - if Nsamp = 0

      - Decrement Nwords

      - If Nwords =0 Go To End Of CCSDS Packet

      - return to Step 2

  - Initialize n=4

  - Initialize ByteCount = 0

  - read next 32 bit word

  - extract compressed value C from byte [ByteCount] of word ; sign extend into 16 bit value

  - Uncompress X[n] = X[n-1] + Slope – C

  - Recalculate Slope = X[n] – X[n-1]

  - decrement Nsamp

    - if Nsamp = 0

      - Decrement Nwords

      - If Nwords =0 Go To End Of CCSDS Packet

      - return to Step 2

  - Increment ByteCount

    - If ByteCount < 4 Go to Step 7-Q

  - Decrement Nwords:

    - If Nwords =0 Go To End Of CCSDS Packet

  - return to Step 7-O

End OF CCSDS Packet:

- If NWORDS and Nsamp not Zero an Error has occurred

- Done with CCSDS Packet

Figure 1 Magnetometer Data Decompression Flowchart

# Magnetometer Data Compression Algorithm

A series of Ndata 16 bit magnetometer data values is compressed using a slope/delta compression scheme. To create a compressed subpacket, the algorithm is:

- Initialize Nsamp(subpacket sample counter)  = 0; Nwords(output 32-bit word counter) = 0

- Store entire first sample in low 16 bits of first output word. Clear bits 16-31.

- Increment Nsamp

- Increment Nwords

- Decrement Ndata – If Zero Go to End of Packet:

- Calculate Slope = X[Nsamp] – X[Nsamp -1]

- If | Slope | > 512  Subacket complete – Go to End of subpacket:

  - Store Slope in bits 0-15 of second word.

  - Increment Nsamp

  - Save indicator that partial word filled (2 bytes stored).

  - Decrement Ndata – If Zero Go to End of Subpacket

  - Predict next value: P = X[Nsamp -1] + Slope

  - Calculate Delta: D = P – X[Nsamp]

  - If  Delta > 127 or if Delta  <  -128 Go to End of Subpacket (note: Nsamp not incremented)

    - Recalculate Slope = X[Nsamp] – X[Nsamp -1] – save for later use

    - Store Delta in next byte of output word; increment partial word filled indicator

    - If 4 bytes stored , increment Nwords; reset partial word filled indicator.

    - Increment Nsamp

    - Decrement Ndata – If Zero Go to End of Subpacket:

    - Go back to Step 7-E

End Of Subpacket:

- Write Nsamp into bits 25-31 of first word of subpacket.

- If partial word indicator not zero Increment Nwords (start new subpacket on new word)

- Return to step 1.

# FAILSAFE Magnetometer data.

When EMFISIS is operating in FAILSAFE Mode, a magnetometer packet containing a single vector will be transmitted once per second.  The packet format is described in Table 5.

Table 8 FAILSAFE Magnetometer Data Packet Format

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 293 (hex) | EMFISIS APID |
| Grouping Flags | 2 | 3 | CCSDS definition of grouping flags: 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | 17 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| Spare | 16 | 0 |  |
| Cal Sate | 1 | 0= off 1= on |  |
| Range | 2 | 0-3 | 00: 256 nT 01: 4096 nT 10: not used 11: 65536 nT |
| Spare | 29 | 0 |  |
| Raw Mag U | 16 | -32768 - 32767 |  |
| Raw Mag V | 16 | -32768 - 32767 |  |
| Raw Mag W | 16 | -32768 - 32767 |  |

- Packets will be generated once per second.

- The time tag will correspond to the data sample.

- Multi-byte data quantities are transmitted Most Significant Byte First in the CCSDS Packet.

- Note the insertion of a 16 bit pad value between the MET Subsecons and the remainder of the packet. This is to simplify the packet formatting in FAILSAFE mode.

# Waves Packets

# Overview

# General Waves Format

Table 9 EMFISIS Waves Packet Format

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 0x2A0 - HFR Waveform 0x2A1 - HFR Specrta 0x2AX – HFR TBD 0x2B0 - WFR Waveform 0x2B2 - WFR Spec. Matrix 0x2Bx - WFR TBD | EMFISIS APID |
| Grouping Flags | 2 | 3 | CCSDS definition of grouping flags: 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | 13 - 4089 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| EMFISIS Grouping Flags | 2 | 0-3 | 0 = intermediate packet 1 = first packet in group 2 = last packet in group 3 = not part of a group |
| Packet Type | 6 | 0-63 | SeeWAVES Packet Type table |
| Offset | 24 | 0 - 162777215 | 32 bit word offset of first data point into grouped data |
| WAVES DATA | Variable | Variable | 12-4080 bytes of data Content will be dependent on Packet type – See Table 10 The first 8 bytes of WAVES Data contain the Waves Header – See Table 11 |

Table 10 WAVES Packet Types

| Packet Type | Contents | Format |
| --- | --- | --- |
| 0 | Raw Data – 32 Bit Integer |  |
| 1 | Raw Data – 32 Bit Float |  |
| 2 | Raw Data – 16 bit Integer |  |
| 3 | Raw Data – 16 Bit Float |  |
| 4 | Waveform – 16 Bit integer |  |
| 5 | FFT – 16 bit Float |  |
| 6 | Spectral Matrix – 16 Bit Float |  |

Table 11 WAVES Header

| Field Description | Size (bits) | Value |
| --- | --- | --- |
| TBD | 60 |  |
| Waveform Channel # | 4 | 0-5 |

# Miscellaneous Packets

# Memory Dump

Table 12 Memory Dump Packet Format

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 281 (hex) | EMFISIS APID |
| Grouping Flags | 2 | 3 | CCSDS definition of grouping flags: 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | 13 - 2057 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| Dump Start Address | 32 | 0-ffffffff |  |
| DATA | N*32 | Variable | Up to 512 32 bit words |

# Waves Dump

The WAVES Dump Packet is a modified form of the generic WAVES Packet  intended for diagnostic use. The 8 Byte WAVES Header is omitted. The data type filed is used to indicated the type of data being dumped. This packet is generated in response to a WavesDump Command and is used to examine memory space within the WAVES FFT engines.

Table 13 WAVES Dump Packet

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 0x288 | EMFISIS APID |
| Grouping Flags | 2 | 3 | CCSDS definition of grouping flags: 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | 13 - 4089 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| EMFISIS Grouping Flags | 2 | 0-3 | 0 = intermediate packet 1 = first packet in group 2 = last packet in group 3 = not part of a group |
| Packet Type | 6 | 0-63 | SeeWAVES Packet Type table |
| Offset | 24 | 0 - 162777215 | 32 bit word offset of first data point into grouped data |
| WAVES DATA | Variable | Variable | 4-4080 bytes of data Content will be dependent on Packet type – See Table 10 Note: there is no WAVES Headed associated with a WAVES Dump Packet. |

# LVPS packet

Table 14 LVPS Packet

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 28f (hex) | EMFISIS APID |
| Grouping Flags | 2 | 3 | CCSDS definition of grouping flags: 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | 73 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| spare | 32 |  | TBD |
| LVPS A/D Channels | 32*16 | 0-ffff | LVPS A/D data |

# Space Weather packet

Table 15 Space Weather Packet

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 0 | 0=telemetry |
| Sec. Hdr. Flag | 1 | 1 | 1=has secondary header |
| Application ID | 11 | 28C(hex) | EMFISIS APID |
| Grouping Flags | 2 | 3 | CCSDS definition of grouping flags: 3 – not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new packet |
| Packet Length | 16 | 73 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) |
| MET SubSeconds | 16 | 0-19999 | 50 microseconds/tic |
| Mag X Component | 16 |  |  |
| Mag Y Component | 16 |  |  |
| Mag Z Component | 16 |  |  |
| Waves Parameter 1 | 16 |  |  |
| Waves Parameter 2 | 16 |  |  |
| Waves Parameter 3 | 16 |  |  |
| Waves Parameter 4 | 16 |  |  |
| Waves Parameter 5 | 16 |  |  |
| Waves Parameter 6 | 16 |  |  |

# OSP

# Commands

# Overview

# Time and Status

Table 16 Time and Status Message

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Packet Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 0x100 | Spacecraft Time and Status Message APID |
| Grouping Flags | 2 | 3 | 3 = Not part of group |
| Sequence Count | 14 | 0-16383 | Increments with each new pkt |
| Packet Length | 16 | 33 | Data size, number of bytes following the “Packet Length” field minus 1. |
| MET Seconds | 32 | 0 – 4294967295 | Mission Elapsed Time (sec), indicates the MET time at the previous 1PPS signal. The time at the next 1PPS can be obtained by adding 1 to this value. |
| Thruster Arm Status | 1 | 0-1 | 0 – Indicates thrusters are not armed and it is safe for instruments to operate. 1 - Indicates that thrusters are armed for an impending propulsion event This bit will be asserted at least 1 spin prior to the firing of thrusters. |
| Instrument Shutdown Warning | 1 | 0-1 | 1=instrument will be shutdown 15 seconds (TBR) after bit is asserted. However, Instruments must be capable of being turned off w/o warning. |
| Eclipse Status | 1 | 0-1 | Indicates the eclipse status of the spacecraft (0=non-eclipse, 1=eclipse). |
| Spacecraft C&DH Reset Status | 1 | 0-1 | Indicates if the C&DH has reset which causes a loss of instrument time-tagged commands. (0=no reset, 1=reset has occurred) |
| Spin Period Validity | 1 | 0-1 | Indicates the validity of the Spin period data (0 = invalid 1 = valid) |
| Spin Phase Validity | 1 | 0-1 | Indicates the validity of the Spin phase data (0 = invalid 1 = valid) |
| Spare | 10 | 0 | Spare bits, always zero (0) |
| Spin Period | 16 | 0 – 65535 | Indicates the spin rate of the spacecraft. The value is provided as an unsigned scaled integer (0 – 65535 = 0 – 20.97 seconds). Hardware timer is a period with 20usec resolution. Most significant bits provide a period with 320usec resolution for spin period to ~21 sec. |
| Spin Phase | 16 | 0 – 65535 | Indicates the spin phase of the spacecraft (at the next 1 PPS) (TBR) The value is provided as an unsigned scaled integer (0 – 65535 = 0 – 360 deg) |
| ECT SSR Status | 8 | 0-255 | Percent full (0 = 0%, 255 = 100%) [TBR] |
| EFW SSR Status | 8 | 0-255 | Percent full (0 = 0%, 255 = 100%) [TBR] |
| EMFISIS Survey SSR Status | 8 | 0-255 | Percent full (0 = 0%, 255 = 100%) [TBR] |
| EMFISIS Burst SSR Status | 8 | 0-255 | Percent full (0 = 0%, 255 = 100%) [TBR] |
| RBSPICE SSR Status | 8 | 0-255 | Percent full (0 = 0%, 255 = 100%) [TBR] |
| RPS SSR Status | 8 | 0-255 | Percent full (0 = 0%, 255 = 100%) [TBR] |
| ECT Shared Data | 48 | [TBD by IDT] | Shared Burst information provide by ECT. <Definition provided by ECT> |
| EFW Shared Data | 32 | [TBD by IDT] | Shared Burst information provide by EFW. <Definition provided by ECT> |
| EMFISIS Shared Data | 32 | [TBD by IDT] | Shared Magnetic Field, Magnetometer Gain and <Burst information provided by EMFISIS> |
| RBSPICE Shared Data | 16 | [TBD by IDT] | Shared Burst information provide by RBSPICE <Definition provided by RBSPICE> |
| SPARE | 16 | 0 | 2 Bytes of spared [Total message length = 40 bytes] |

# EMFISIS Telecommands

Table 17 Generic EMFISIS Telecommand

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 640 - 703 (dec) 280- 2BF (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 0 – 3 | CCSDS Grouping Flags: 0 = Intermediate Packet 1 = First Packet in group 2 = Last packet in group 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet (1) |
| Packet Length | 16 | 3 - 249 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | Variable | EMFISIS data portion of command packet as defined by instrument (2) |
| Flags | 8 |  |  |
| Command Data | 0-244 byte |  |  |
| Notes: (1) The sequence counts on commands do not increment smoothly when there is a mixture of real-time and time-tagged instrument commands. Each of these command sources maintains it own command sequence count. (2) Command packets will not span ITFs and ITFs will not span 1 PPS boundaries |  |  |  |

# NOP

Table 18 NOP

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 5 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 0 |  |
| Flags | 8 | 0 | Always 0 |

# Reset

Table 19 Reset

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 5 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 1 |  |
| Flags | 8 |  | 0x0 – normal RESET 0x1 – Boot to AHBROM 0x2 – Boot to alternate Address |
| Checksum | 16 |  | XOR of 16 bits containing FC and Flags with 0xBEEF |

# Write Enable

Table 20 Write Enable

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 5 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 2 |  |
| Flags | 8 |  | TBD |
| Checksum | 16 |  | XOR of 16 bits containing FC and Flags with 0xBEEF |

# Peek

Table 21 Peek

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 9 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 3 |  |
| Flags | 8 | 0 | Always 0 |
| Address | 32 |  | Address to be read. Contents of Address are reported via a Memory Dump Packet |

# Poke

Table 22 Poke

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 13 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 4 |  |
| Flags | 8 |  | 0=normal POKE 1=OR 2=XOR 3=AND |
| Address | 32 |  | Address to be written to. After write, the contents of the Address are reported via a Memory Dump Packet |
| Value | 32 |  | Value to use in operation on Address |

# Jump

Not implemented

# MagPacketType

Table 23 MagPacketType

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 5 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 0x20 |  |
| Packet type | 8 |  | 0=none 1=Uncompressed Only 2=Comressed Only 3= Both |

# UploadAbort

Table 24 UploadAbort

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 5 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 0x7c |  |
| Flags | 8 |  | TBD |

# UploadData

Table 25 UploadData

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 9-xxx | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 0x8d |  |
| unused | 8 | 0 | Always 0 |
| Data | N*32 |  | N 32 bit words to be loaded to address specified by Upload Start Command. Multiple Uplad Data commands may be send to complet the upload. |

# Upload Start

Table 26 UploadStart

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 13 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 0x80 |  |
| Unused | 8 | 0 | Always 0 |
| Upload Start Address | 32 |  | Address to begin Load |
| Load Length | 32 |  | Number of 32-bit words in Load. |

# MemoryDump

Table 27 MemoryDump

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 13 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 0x80 |  |
| Flags | 8 |  | TBD |
| Dump Start Address | 32 |  | Address to begin Memory Dump |
| Dump Length | 32 |  | Number of 32-bit words to dump. |

# WavesDump

Table 28 WavesDump

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 13 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 0x81 |  |
| Type | 8 | 0 | Waves Data Type – See table 10 |
| Dump Start Address | 32 |  | Start Address for Dump of WAVES data |
| Dump Length | 32 |  | Number of 32-bi words to Dump. |

# EnableWavesInterrupt

Table 29 EnableWavesInterrupt

| Field Description | Size (bits) | Value | Note |
| --- | --- | --- | --- |
| CCSDS Version | 3 | 0 | Value is always zero (0) |
| Pkt Type | 1 | 1 | 1=telecommand |
| Sec. Hdr. Flag | 1 | 1 | 1=has sec. header |
| Application ID | 11 | 280 - (hex) | EMFISIS Instrument Allocated command message APID |
| Grouping Flags | 2 | 3 | CCSDS Grouping Flags: 3 = Not part of group |
| Sequence Count | 14 | 0 - 16383 | Increments with each new packet |
| Packet Length | 16 | 13 | Data size, number of bytes following the “Packet Length” minus 1, (value is always ODD) |
| MET Seconds | 32 | 0 – 4294967295 | Packet Time (sec) = Time-Tag for stored commands, A value of 0 indicates the command is to be sent in real time. |
| Function Code | 8 | 0x83 |  |
| WAVES interrupt MASK | 8 | 0 | Bit 0 = IRQA – 0=disable 1= enable Bit 1 = IRQB – 0=disable 1= enable Bit 2 = IRQC – 0=disable 1= enable Bit 3 = IRQD – 0=disable 1= enable |